# INDEX

## SECTION A

QUESTION 1

1.1.

1.2

1.3.

QUESTION 2

1 .

# SECTION A

QUESTION 1:

ANSWER THE FOLLOWING QUESTIONS:

1.1. Design Subnet for FIVE Department at Eduvos .

- Science (SC) - 100 hosts

- Arts (AR) - 50 hosts

- Technology (TE) - 200 hosts

- Commerce (CO) - 150 hosts

- Administration (AD) - 25 hosts

- We’ll create a separate subnet for each Department...

Binary Table :

- Private IP (Internet Protocol) Address: 192.168.0.0/16

| Department | Subnet Address | Subnet Mask |
| --- | --- | --- |
| Science (SC) | 192.168.1.0/24 | 255.255.255.0 |
| Arts (AR) | 192.168.2.0/26 | 255.255.255.192 |
| Technology (TE) | 192.168.3.0/23 | 255.255.254.0 |
| Commerce (CO) | 192.168.5.0/24 | 255.255.255.0 |
| Administration (AD) | 192.168.6.0/27 | 255.255.255.224 |

1.2. Determing Subnet Masks

- Science

= 11111111.11111111.111111111.000000000

= 255.255.255.0

- Arts

= 111111111.111111111.111111111.11000000

= 255.255.255.192

- Technology

= 11111111.11111111.11111110.00000000

= 255.255.254.0

- Commerce

= 11111111.1111111.11111111.000000000

= 255.255.255.0

- Administration

= 11111111.11111111.11111111.11100000

= 255.255.255.224

1.3. Calculate the number of Subnets and Hosts per Subnets

- Science (SC)

= 32 – 24

= 8 bits

= 2^8 – 2

= 254 host per Subnet.

- Arts (AR)

= 32 – 26

= 6 bits

= 2^6 – 2

= 62 host per Subnet.

- Technology (TE)

= 32 – 23

= 2^9 – 2

= 510 host per Subnet.

- Commerce (CO)

= 32 – 24

= 8 bits

= 2^8 – 2

= 254 host per Subnet.

- Administration

= 32 – 27

= 5 bits

= 2^5 – 2

= 30 host per Subnet

- Departments

- Number of Subnets

- Number of Host per Subnet

- SC | 1 | 254

- AR | 1 | 62

- TE | 1 | 510

- CO | 1 | 254

- AD | 1 | 30

# QUESTION 2

- HOW DOES THE DESIGN AND IMPLEMENTATION OF A SUBNETTED PRIVATE NETWORK FOR FIVE BRANCHES IMPACT NETWORK PERFORMANCE, SECURITY, AND SCALABILITY?

The designing and implementation of a subnetted private network for five branches of Eduvos can have an impact, negatively and positively. It has an impact on network performance, security and scalability.

## Network Performance:

- Negative impacts;

- Increased latency: meaning that routing traffic between subnet can cause more latency, affecting applicationsthat need low latency.

- More complex routing: meaning that more complex routing configurations requires subnetting, if not applied properly it can lead to routing errors.

- Positive impacts;

- Reduced Network Congestion: Subnetting helps localize traffic with each subnet by dividing the network into smaller subnets. The amount of traffic on each subnet is reduced, which leads the improved network performance.

- Improved Speed: meaning that if a problem occurs on one subnet, it will not affect the other subnets, it reduces the load on the network devices and improves speed and performance.

## Security:

- Positive impacts;

- Enhanced Security: Subnetting allows it to have better control over network into subnet, it is reduced and makes it hard for potential attackers to move laterally across the network.

- Improved access control: meaning that with subnetting, we can use Access Control Lists (ACLs) and other security measures to control, making it easier to restrict sensitive areas of the network or data.

- Negative impacts;

- Increased complexity: submetting causes more complexity, it can cause security misconfiguration if not managed correctly.

- More network devices: it requires more devices for a subnet (e.g routers, switches) which can increase the attack surface if not secured well.

## Scalability:

-  Negative impacts;

- Increased management complexity: Network growth can be challenging because subnetting needs more complex network management.

- Potential for subnet exhaustion: subnetting can cause subnet exhaustion if not properly handled and making it hard to add new devices and branches.

-Positive impacts;

- Efficient IP Address Management: This allows use of IP address, dividing larger network into smaller subnets. You allocate an IP address without wastage.

- Easier expansion: As growth occurs in the branches it makes more easier to add new branches or departments with subnetting.

In conclusion, the design and implementation of a subnetted private network for Eduvos' five branches can have both positive and negative impacts on network performance, security, and scalability. Proper planning, configuration, and management are essential to minimize the negative impacts and maximize the benefits of subnetting.

# REFERENCES:

Subratie, K., Aditya, S., Daneshmand, V., Ichikawa, K. and Figueiredo, R., 2020. On the design and implementation of IP-over-P2P overlay virtual private networks. IEICE Transactions on Communications, 103(1), pp.2-10. ieice.org

Das, T.S., Ghosal, P., Chatterjee, N., Nath, A., Banerjee, A. and Khastagir, S., 2020. Application of logical sub-networking in congestion-aware deadlock-free SDmesh routing. ACM Transactions on Embedded Computing Systems (TECS), 19(4), pp.1-26. researchgate.net

Gentile, A. F., Fazio, P., & Miceli, G., 2021. A Survey on the Implementation and Management of Secure Virtual Private Networks (VPNs) and Virtual LANs (VLANs) in Static and Mobile Scenarios. Telecom. mdpi.com